*

Hall Effect Example Code

* Connect a 1K resistor from 5V to pin 2.

* Connect a 0.1uF capacitor between pin 2 and ground.

* Connect the hall effect output pin to pin 2.

* Connect power and ground from the Arduino to the appropriate hall effect sensor pins.

This example code is in the public domain.

*/

int sensorPin1 = 2;  // sensor on pin 2

int sensorState1;    // variable for reading the sensor

void setup() {

//set pin 2 as an input

pinMode(sensorPin1, INPUT);

//begin communication to the serial monitor

Serial.begin(9600);

}

void loop(){

// read the sensor pin

sensorState1 = digitalRead(sensorPin1);

// check if the sensor is activated.

// if it is, the sensor goes LOW:

if (buttonState1 == LOW) {

// print to the serial monitor:

Serial.println("magnet present");

}

}